# Introduction

This document gives a brief description of IBIT (Initiated Build-In-Test) for all DR64 devices connected to I2C  bus

# Temperature Sensor (TMP75B-Q1)

## Interrupt test

for all Temp devices:

Save original values of Low/High limit registers

Set configuration register to support interrupt (0x02)

# --- Check T-High ALERT

Set T-High low-then-normal temperature (+5c). T-Low stay at -40c

Print "Pass"/"Fail" according to value of PC_04 GPIO (low=pass)

Clear int. line (by reading register)

# --- Check T-Low ALERT

Set T-Low to +60c & T-High to +65c

Print "Pass"/"Fail" according to value of PC_04 GPIO (low=pass)

Clear int. line (by reading register)

Restore original values of T-Low/T-High registers (read before test)

# Voltage Monitor (TPS389006/08-Q1)

## Limit Test

## Interrupt test

for all VMON devices:

for all channels in device I:

# Low/High Voltage limit

read dev[i].chan[j] HV/LV limits

read dev[i].chan[j] voltage read (Vij)

# --- HV test

Set HVij = Vij – DELTA;  # DELTA > channel tolerance

Read Int line (PK_07) and print (active = "PASS")

Restore original HVij

Read Int line (PK_07) and print (non-active = "PASS")

// --- LV test

Set LVij = Vij + DELTA;  # DELTA > channel tolerance

Read Int line (PK_07) and print (active = "PASS")

Restore original LVij

Read Int line (PK_07) and print (non-active = "PASS")

# GPIO Expender (TCA9539-Q1)

## Interrupt test